- Plug in the Time Card to your Linux machine PCIe slot and turn on the system

- Once the PC is booted up, open a terminal

- Run lspci | grep Meta      , verify you see a device listed like this

- Run modprobe ptp_ocp    , then dmesg | grep ptp_ocp    . Verify you see messages from ptp_ocp in the output

- To verify the GPS module, run sudo tio -b 115200 /dev/ttyS5     , using the /dev/ttyS device listed as GNSS: in the dmesg output, as shown here

- You should see messages scrolling , this is the binary output from the GPS receiver which is good

- Use ctrl+t then q to quit out of tio

- Verify the GPS fully

  - If this is the first time doing this, run these commands to install

    - sudo yum install python3

    - sudo yum install python3-tkinter

    - sudo python3 -m pip install --upgrade pip

    - sudo python3 -m pip install --upgrade Pillow

    - sudo python3 -m pip install pygpsclient

- Run su to enter root, enter your root password

- Run pygpsclient , a GUI will pop up

- Select the GNSS /dev/ttyS device used earlier in TIO , in my example it is /dev/ttyS5 , and change the Rate bps to 115200

- Press the USB/UART button to connect

- If you have a GPS antenna connected, you should see full GPS information from the device, like this example. If no GPS antenna is connected, you should at least see some <UBX messages in the text box. This will fully verify the GPS operation

- Close out of the PyGPSClient

- To verify all the SMAs are working properly , run the following commands

  - sudo echo OUT: PHC >> /sys/class/timecard/ocp0/sma1

  - sudo echo OUT: PHC >> /sys/class/timecard/ocp0/sma2

  - sudo echo OUT: PHC >> /sys/class/timecard/ocp0/sma3

  - sudo echo OUT: PHC >> /sys/class/timecard/ocp0/sma4

- Measure the four SMA outputs on an oscilloscope. You should see a 1Hz signal coming out of every SMA

- To verify the frequency signal, run the following commands

  - sudo echo OUT: 10Mhz >> /sys/class/timecard/ocp0/sma1

  - sudo echo OUT: 10Mhz >> /sys/class/timecard/ocp0/sma2

  - sudo echo OUT: 10Mhz >> /sys/class/timecard/ocp0/sma3

  - sudo echo OUT: 10Mhz >> /sys/class/timecard/ocp0/sma4

- Measure the four SMA outputs on an oscilloscope. You should see a 10MHz signal coming out of every SMA

This verifies the FPGA, the oscillator, and the GPS, verifying the time card is fully operational.